Preventive Maintenance Checklist

Regular maintenance helps maximize reliability, safety, and the service life of your Magnum Mobile unit.

## MONTHLY

☐ Lubricate all moving parts:

- Entry steps

- Hinges

- Latches

- Locks

## EVERY 3 MONTHS

☐ Review and complete all recommended Ford/GM chassis maintenance, including oil changes

☐ Review and complete all recommended Generator maintenance (including oil change every 150 hours for gas models, every 250 hours for diesel)

☐ Inspect generator compartment for debris and airflow restrictions

☐ Clean rooftop A/C filters

☐ Sanitize fresh water tank

☐ Inspect optional equipment and clean filters as required (oxygen concentrator, lift systems, etc.)

## EVERY 6 MONTHS

☐ Inspect roof sealant around:

- Roof perimeter

- Solar panel mounts

- Vents

- Roof penetrations

☐ Inspect silicone seals around:

- Cabinets and cages

- Countertops

- Flooring edges

- Interior partitions

☐ Replace any cracked, missing, or worn sealant.  Roof sealant is SikaFlex 715.

☐ Inspect LP gas system for leaks (if equipped)

## ANNUALLY

☐ Inspect undercarriage for rust, corrosion, or loose hardware.  This applies to both chassis and trailers.

☐ Inspect rear box mounting bolts and verify proper torque per manufacturer specifications

☐ Inspect plumbing fittings and water lines for leaks

☐ Inspect all safety equipment and operational systems

## RECOMMENDED CLEANING PRODUCTS

- Formula 409

- Simple Green All-Purpose Cleaner

- Lysol Pro Kitchen Cleaner

- Zep Commercial Neutral Floor Cleaner, Simple Green, Naturally Green cleaners for Altro woodgrain pattern floor

- Armstrong “Shine Keepger” to reseal raised coin flooring

✖ Do Not Use Rubbing Alcohol on interior surfaces

## IMPORTANT REMINDERS

☐ Use a 50-Amp Surge Protector with Circuit Analyzer when connecting to shore power (or a 30-Amp version if the unit has a 4k generator)

☐ Park in secure, well-lit areas whenever possible to help deter catalytic converter theft

☐ During extreme heat, maximize airflow around the generator compartment for improved performance

☐ Winterize water system before exposure to freezing temperatures

## LONG-TERM OWNERSHIP PLANNING

Typical replacement intervals:

• Auxiliary Battery: 2–4 Years

• Battery Charger: 5–7 Years

• Roof A/C Unit: 5–10 Years

• Generator: 5–10+ Years

•Chassis: Tires (+/- 30,000), Battery 2-4 years

•With proper care and maintenance, your unit can last well over 15 years.  We are happy to help when it is time to plan for a new one!

Actual service life depends on usage, operating conditions, and maintenance practices.For maintenance questions, replacement parts, troubleshooting, or training resources, please contact Magnum Mobile at info@magnummobilesv.com or 602-715-2814.

Thank you for choosing Magnum Mobile!

www.magnummobilesv.com